Introduction to Robotics — PRACTICE Final Exam

Course: Introduction to Robotics for AI & Data Science  |  Code: 0142351

Instructor: Dr. Akram Fatayer  |  Duration: 2 hours  |  Total: 40 points

This practice exam mirrors the structure of the real final. The exam is organized by the robot's own pipeline — Sensing, Localization, Mapping & Planning, Tracking & Control, and Integration — so it walks the same path you followed all semester.

Instructions:

Closed book. A formula sheet is provided.

Answer all questions. Show your work on calculation problems for partial credit.

For multiple choice, circle the single best answer.

Section A — Foundations & Sensing   (7 points)

Multiple Choice (3 x 1.5 = 4.5 points)

A1. Which of the following is NOT one of the three components of the Sense–Plan–Act paradigm?

A) Sensing

B) Planning

C) Power supply

D) Acting

A2. A wheeled robot that can drive forward and turn, but cannot move directly sideways, is subject to a:

A) holonomic constraint

B) non-holonomic constraint

C) kinematic singularity

D) steady-state error

A3. Which sensor is most appropriate for directly measuring the distance to nearby obstacles for mapping?

A) Wheel encoder

B) IMU

C) LiDAR

D) Battery monitor

Short Answer (2.5 points)

A4. Define the robot 'pose' and explain why orientation (theta) matters for navigation, not just position (x, y). (2-3 sentences)

_______________________________________________________________________________________________

_______________________________________________________________________________________________

_______________________________________________________________________________________________

Section B — Localization   (9 points)

Multiple Choice (2 x 1.5 = 3 points)

B1. Odometry drift occurs because:

A) the battery voltage drops over time

B) small measurement errors accumulate over time

C) the map resolution is too low

D) the goal position keeps changing

B2. In a Kalman filter, the PREDICT step uses the motion model to project the state forward. What happens to the estimate's uncertainty during predict (before any measurement)?

A) it shrinks

B) it grows

C) it stays exactly the same

D) it becomes zero

Short Answer (3 points)

B3. A robot has wheel odometry (smooth but drifts over time) and a noisy GPS-style sensor (jumpy but does not drift). Explain why fusing both with a Kalman filter gives a better pose estimate than using either one alone. (3-4 sentences)

_______________________________________________________________________________________________

_______________________________________________________________________________________________

_______________________________________________________________________________________________

_______________________________________________________________________________________________

Problem B4 — Kalman Filter, one step (3 points)

A robot moves along a line. Its state is [position, velocity], and it uses a constant-velocity motion model with time step dt = 1 s. Use the Kalman filter equations from the formula sheet.

Given:

Current state estimate: position = 2.0 m, velocity = 0.5 m/s

Predicted covariance has already been computed: P_pred[position] = 1.5

A new position measurement arrives: z = 2.0 m

Measurement noise: R = 0.5

a) (1 pt) Compute the predicted position after the predict step.

_______________________________________________________________________________________________

b) (1 pt) Compute the innovation y.

_______________________________________________________________________________________________

c) (1 pt) Compute the innovation covariance S, the Kalman gain K for position, and the corrected position.

_______________________________________________________________________________________________

_______________________________________________________________________________________________

Section C — Mapping & Planning   (12 points)

Multiple Choice (3 x 1.5 = 4.5 points)

C1. In an occupancy grid, a cell value of 1 typically represents:

A) free space

B) an obstacle

C) unknown / unexplored

D) the goal

C2. Why do we INFLATE obstacles in the occupancy grid before planning?

A) to make the map look clearer

B) to account for the robot's physical size so its body does not clip obstacles

C) to make A* run faster

D) to increase the grid resolution

C3. A* would be a POOR choice (and RRT a better one) for which of the following?

A) a 2-D warehouse floor grid

B) planning for a 6-joint robot arm in high-dimensional space

C) a small known static map

D) finding the shortest path on a road network

Short Answer (2.5 points)

C4. What does it mean for an A* heuristic to be ADMISSIBLE, and why does admissibility matter? (2-3 sentences)

_______________________________________________________________________________________________

_______________________________________________________________________________________________

_______________________________________________________________________________________________

Problem C5 — A* Path Planning (5 points)

Consider the 5x5 grid below. 'S' is the start at (row 0, col 0), 'G' is the goal at (row 4, col 4), and 'X' are obstacles. Use 4-connected movement (up/down/left/right), step cost 1, and the Manhattan heuristic.

col0 col1 col2 col3 col4
      +----+----+----+----+----+
  row0|  S |    |    |    |    |
      +----+----+----+----+----+
  row1|    |  X |  X |    |    |
      +----+----+----+----+----+
  row2|    |  X |    |    |    |
      +----+----+----+----+----+
  row3|    |    |    |  X |    |
      +----+----+----+----+----+
  row4|    |    |    |    |  G |
      +----+----+----+----+----+

a) (1 pt) Compute h(S), the Manhattan heuristic at the start.

_______________________________________________________________________________________________

b) (2 pts) After expanding S, list its valid neighbours and compute f(n) for each.

_______________________________________________________________________________________________

_______________________________________________________________________________________________

_______________________________________________________________________________________________

c) (2 pts) Give one optimal path from S to G and state its total cost.

_______________________________________________________________________________________________

_______________________________________________________________________________________________

_______________________________________________________________________________________________

Section D — Tracking & Control   (10 points)

Multiple Choice (2 x 1.5 = 3 points)

D1. Which PID term is primarily responsible for eliminating steady-state error?

A) Proportional (P)

B) Integral (I)

C) Derivative (D)

D) none of them

D2. In Pure Pursuit, making the look-ahead distance Ld too SMALL tends to cause:

A) cutting corners on turns

B) oscillation / jerky motion

C) the robot to stop

D) the robot to ignore the path

Short Answer (3 points)

D3. A path tracker tries to drive two errors to zero: cross-track error (CTE) and heading error. Explain the difference between them, and why driving only CTE to zero is not enough. (3-4 sentences)

_______________________________________________________________________________________________

_______________________________________________________________________________________________

_______________________________________________________________________________________________

_______________________________________________________________________________________________

Problem D4 — Pure Pursuit (4 points)

A mobile robot at position (1.0, 1.0) has heading 0 degrees (facing the +x direction) and linear velocity v = 0.5 m/s. The look-ahead point on the path is at (3.0, 2.0).

a) (1.5 pts) Compute the angle to the look-ahead point and the heading error alpha. (Hint: heading is 0, so alpha equals the angle to the point.)

_______________________________________________________________________________________________

_______________________________________________________________________________________________

b) (1.5 pts) The look-ahead distance is Ld = sqrt(5) ~ 2.236 m. Compute the curvature kappa.

_______________________________________________________________________________________________

_______________________________________________________________________________________________

c) (1 pt) Compute the angular velocity omega.

_______________________________________________________________________________________________

Section E — System Integration   (2 points)

E1. In the full autonomy pipeline (Sense -> Localize -> Map -> Plan -> Track), explain how a poor result in ONE early stage damages a later stage. Give one concrete example. (2-3 sentences)

_______________________________________________________________________________________________

_______________________________________________________________________________________________

_______________________________________________________________________________________________

_______________________________________________________________________________________________

End of Practice Exam